YSF2DMR – (FreeDMR) with MMDVMHost running YSFGateway.

Basic overview of how it works on GB3HB

edit ysf2dmr.ini to suit required port /setup

run ysf2dmr

edit ysfhosts.txt to add ysf2dmr entry & port

Example ysf2dmr.ini (You need to edit the highlighted parts below with your details)

[Info]

RXFrequency=434975000

TXFrequency=433375000

Power=3

Latitude=0.0

Longitude=0.0

Height=0

Location=Anywhere, UK

Description=C4FM Repeater to FreeDMR TG

URL=www.google.co.uk

[YSF Network]

Callsign=M1ABC

Suffix=RPT

DstAddress=127.0.0.1

DstPort=42000

LocalAddress=127.0.0.1

LocalPort=42014

EnableWiresX=0

Daemon=0

[DMR Network]

Id=235xxx

StartupDstId=2350

# For TG call: StartupPC=0

StartupPC=0

# Address=hotspot.uk.freedmr.link

Address=repeater.uk.freedmr.link

Port=62031

Jitter=300

EnableUnlink=0

TGUnlink=4000

PCUnlink=0

# Local=62032

Password=passw0rd

Options=TS1=;TS2=2350;DIAL=0;TIMER=0;VOICE=0

TGListFile=TGList-DMR.txt

Debug=0

[DMR Id Lookup]

File=DMRIds.dat

Time=24

[Log]

# Logging levels, 0=No logging

DisplayLevel=0

FileLevel=1

FilePath=/var/log/????/

FileRoot=YSF2DMR

[aprs.fi]

Enable=0

Server=euro.aprs2.net

Port=14580

Password=99999

APIKey=Apikey

Refresh=240

Description=FreeDMR enabled Fusion Repeater

Example of line entry in YSFHosts.txt

02350;FreeDMR-TG2350;FreeDMR-TG2350;127.0.0.1;42014;001;http://

Notes

Remember to restart ysfgateway after adding line to ysfhosts.txt file otherwise it will not show up in wires-x list.

To do multiple YSF2DMR TG's you need to run multiple instances of ysf2dmr each with a unique port number (the example above is using 42014) and DMR ID (ie. Add an SSID to each call)

In the example above the YSF direct dial is 02350. On an FTM-100, once in wires-x mode, this can be entered by pressing and holding the # key until it appears on the dial screen and then entering 02350#. This is purely local to the hotspot/gateway you are working as that is the only place that holds the correct ipaddress and port number for your specific ysf2dmr.